DOC-001

DRUG MASTER FILE (DMF)

MODULE 3.2.S: DRUG SUBSTANCE (API)

Version:  1.0
Date:  [To be determined]
Author: Stefan Schönwälder

This document is for demonstration and training purposes only.

## MODULE 3.2.S: DRUG SUBSTANCE (API)

## Fampridin (4-Aminopyridine)

Submission to: Pharmaceuticals and Medical Devices Agency (PMDA), Japan

DMF Type: Type II Drug Master File

Format: Common Technical Document (CTD)

DMF Number: DMF-FAM-001

Submission Date: [To be determined]

Applicant: [Manufacturing Site Name]

Address: [Site Address]

Document Control:

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | 04.12.2025 | Regulatory Affairs | Initial DMF for PMDA submission |

## TABLE OF CONTENTS

MODULE 3.2.S: DRUG SUBSTANCE

- 3.2.S.1 General Information

- 3.2.S.1.1 Nomenclature

- 3.2.S.1.2 Structure

- 3.2.S.1.3 General Properties

- 3.2.S.2 Manufacture

- 3.2.S.2.1 Manufacturer(s)

- 3.2.S.2.2 Description of Manufacturing Process and Process Controls

- 3.2.S.2.3 Control of Materials

- 3.2.S.2.4 Controls of Critical Steps and Intermediates

- 3.2.S.2.5 Process Validation and/or Evaluation

- 3.2.S.2.6 Manufacturing Process Development

- 3.2.S.3 Characterization

- 3.2.S.3.1 Elucidation of Structure and Other Characteristics

- 3.2.S.3.2 Impurities

- 3.2.S.4 Control of Drug Substance

- 3.2.S.4.1 Specification

- 3.2.S.4.2 Analytical Procedures

- 3.2.S.4.3 Validation of Analytical Procedures

- 3.2.S.4.4 Batch Analyses

- 3.2.S.4.5 Justification of Specification

- 3.2.S.5 Reference Standards or Materials

- 3.2.S.6 Container Closure System

- 3.2.S.7 Stability

- 3.2.S.7.1 Stability Summary and Conclusions

- 3.2.S.7.2 Post-approval Stability Protocol and Stability Commitment

- 3.2.S.7.3 Stability Data

---

## 3.2.S.1 GENERAL INFORMATION

### 3.2.S.1.1 NOMENCLATURE

International Nonproprietary Name (INN): Fampridine (WHO)

Alternative INN: Fampridin (German, European)

Chemical Name (IUPAC): 4-Aminopyridine

CAS Registry Number: 504-24-5

Molecular Formula: C₅H₆N₂

Molecular Weight: 94.11 g/mol

Compendial Status:

- DAC (Deutscher Arzneimittel-Codex): Monograph available

- Japanese Pharmacopoeia (JP): Not currently listed (to be established)

Pharmacological Class: Potassium Channel Blocker

Therapeutic Use: Treatment of walking impairment in patients with Multiple Sclerosis

---

### 3.2.S.1.2 STRUCTURE

Structural Formula:

NH₂
         |
    N — C — C
    |       |
    C — C — C
        |
        H

Chemical Structure (2D):

- Pyridine ring with amino group at position 4

Stereochemistry: Not applicable (no chiral centers)

Molecular Formula: C₅H₆N₂

Structural Confirmation:

The structure of Fampridin has been confirmed by the following analytical techniques:

- Infrared Spectroscopy (IR): Characteristic absorption bands consistent with amino group and aromatic ring

- Nuclear Magnetic Resonance (¹H-NMR and ¹³C-NMR): Confirmed substitution pattern on pyridine ring

- Mass Spectrometry (MS): Molecular ion peak at m/z = 94.11 (consistent with molecular formula)

- Elemental Analysis: C, H, N content consistent with C₅H₆N₂

---

### 3.2.S.1.3 GENERAL PROPERTIES

Physical Form: White to off-white crystalline powder

Solubility:

- Water: Freely soluble (>100 mg/mL at 25°C)

- Ethanol: Soluble (~50 mg/mL at 25°C)

- Methanol: Soluble

- Acetone: Sparingly soluble

- Dichloromethane: Practically insoluble

pKa: 9.2 (basic amine group)

Melting Point: 158-160°C (literature value)

Partition Coefficient (log P): -0.20 (octanol/water at pH 7.4) – Hydrophilic compound

Hygroscopicity: Non-hygroscopic (water uptake <0.2% at 25°C/80% RH)

Polymorphism: No polymorphs identified. Single crystalline form (Form I) consistently obtained from manufacturing process.

---

## 3.2.S.2 MANUFACTURE

### 3.2.S.2.1 MANUFACTURER(S)

API Manufacturer:

- Name: [Manufacturing Site Name]

- Address: [Site Address, City, Country]

- GMP Certification: EU GMP Certificate issued by [Authority], Certificate No. [XXX], valid until [Date]

- Manufacturing Authorization: [Authorization Number]

Regulatory Status:

- The manufacturing site is inspected and approved for API production under EU GMP guidelines.

- PMDA pre-approval inspection requested (pending).

---

### 3.2.S.2.2 DESCRIPTION OF MANUFACTURING PROCESS AND PROCESS CONTROLS

#### 3.2.S.2.2.1 MANUFACTURING PROCESS OVERVIEW

Process Type: Chemical Synthesis

Number of Steps: 6 main unit operations

Batch Size: 100 kg (commercial scale)

Process Flow Diagram:

Raw Materials
     ↓
[Step 1: REACTION] → Synthesis Reactor R-101
     ↓
[Step 2: WORKUP] → Neutralization, Extraction
     ↓
[Step 3: CRYSTALLIZATION] → Purification
     ↓
[Step 4: FILTRATION] → Solid/Liquid Separation
     ↓
[Step 5: DRYING] → Vacuum Dryer VD-101
     ↓
[Step 6: MILLING] → Particle Size Control
     ↓
[PACKAGING] → Polyethylene-lined Drums
     ↓
API (Fampridin)

---

#### 3.2.S.2.2.2 DETAILED MANUFACTURING PROCESS

STEP 1: REACTION (SYNTHESIS)

Equipment: Glass-lined reactor R-101 (500 L capacity)

Starting Materials:

- Starting Material A: [Chemical Name], CAS [XXX-XX-X]

- Starting Material B: [Chemical Name], CAS [XXX-XX-X]

- Solvent: Ethanol (Ph. Eur. grade)

Procedure:

1. Charge Starting Material A ([XX] kg) and Starting Material B ([XX] kg) to reactor

2. Add ethanol ([XX] L)

3. Heat to 80 ± 2°C under nitrogen atmosphere

4. Maintain temperature for 4.0 ± 0.5 hours with agitation

5. Monitor pH: Target 7.0 ± 0.5

Critical Process Parameters (CPPs):

- Reaction Temperature: 80 ± 2°C (monitored continuously via calibrated temperature probe)

- Reaction Time: 4.0 ± 0.5 hours

- pH: 7.0 ± 0.5

In-Process Control (IPC):

- IPC-1: HPLC purity at end of reaction: ≥95%

- If purity <95%: Extend reaction time (maximum 5 hours total)

Theoretical Yield (Step 1): ~95%

---

STEP 2: WORKUP (NEUTRALIZATION AND EXTRACTION)

Equipment: Workup reactor R-102 (300 L), Separator vessel

Procedure:

1. Transfer reaction mixture to workup reactor R-102

2. Cool to 25 ± 5°C

3. Adjust pH to 6.5-7.5 using [Acid/Base]

4. Add extraction solvent ([XX] L)

5. Mix for 30 minutes, allow phase separation

6. Separate aqueous and organic layers

7. Collect organic layer containing Fampridin

Critical Process Parameters:

- pH after neutralization: 6.5-7.5 (monitored via calibrated pH meter)

In-Process Control:

- IPC-2: pH verification: 6.5-7.5

- Action if OOS: Adjust with acid or base

---

STEP 3: CRYSTALLIZATION (PURIFICATION)

Equipment: Crystallizer C-101 (jacketed vessel, 400 L)

Procedure:

1. Transfer organic phase to crystallizer

2. Concentrate by distillation under vacuum (remove ~50% solvent)

3. Cool to 10 ± 5°C to induce crystallization

4. Hold at crystallization temperature for 2 ± 0.5 hours

5. Age crystals with agitation

Critical Process Parameters:

- Crystallization Temperature: 10 ± 5°C

- Crystallization Hold Time: 2 ± 0.5 hours

In-Process Control:

- IPC-3: HPLC purity after crystallization: ≥98%

- If purity <98%: Recrystallize

Expected Yield (Step 3): ~90% (from Step 1 input)

---

STEP 4: FILTRATION

Equipment: Nutsche filter F-101 (filter area: 1 m²)

Procedure:

1. Filter crystals under vacuum

2. Wash with cold ethanol ([XX] L)

3. Dry filter cake under vacuum for 30 minutes

In-Process Control: Visual inspection (crystals white to off-white)

---

STEP 5: DRYING

Equipment: Vacuum dryer VD-101 (tray dryer, 200 L capacity)

Procedure:

1. Transfer wet filter cake to vacuum dryer

2. Dry at 60 ± 5°C under vacuum <50 mbar

3. Dry until Loss on Drying (LOD) ≤0.50%

4. Typical drying time: 8-12 hours

5. Cool to room temperature under nitrogen

Critical Process Parameters:

- Drying Temperature: 60 ± 5°C

- Vacuum Level: <50 mbar

In-Process Control:

- IPC-4: Loss on Drying (LOD) ≤0.50%

- Action if >0.50%: Continue drying

Expected Yield (Step 5): ~88% (from Step 1 input)

---

STEP 6: MILLING

Equipment: Impact mill M-101

Procedure:

1. Mill dried API to achieve target particle size

2. Pass through sieve (if applicable)

Critical Process Parameter:

- Target Particle Size Distribution: D50: 20-80 μm

In-Process Control:

- IPC-5: Particle size distribution by laser diffraction: D50: 20-80 μm

- If out of range: Re-mill or blend with appropriate batch

Final Yield (Overall): 85-95% (from Step 1 starting materials)

---

STEP 7: PACKAGING

Equipment: Packaging area (ISO 8 cleanroom)

Procedure:

1. Transfer milled API to polyethylene-lined drums (25 kg per drum)

2. Close drums with secure lids and tamper-evident seals

3. Label drums with:

- Product name: Fampridin

- Batch number

- Manufacturing date

- Quantity

- Storage conditions: "Store at 15-25°C, protect from light and moisture"

Packaging Material: Food-grade polyethylene liner, steel drum (exterior)

---

### 3.2.S.2.3 CONTROL OF MATERIALS

#### 3.2.S.2.3.1 RAW MATERIALS

All raw materials are purchased from qualified suppliers and tested per specifications before use.

Starting Material A:

- Specification: [Details of identity, purity, impurity limits]

- Supplier: [Supplier Name, Country]

- Testing: Identity (IR), Assay (HPLC), Impurities (HPLC), Water Content (KF)

Starting Material B:

- Specification: [Details]

- Supplier: [Supplier Name, Country]

- Testing: Identity, Assay, Impurities

Solvents (Ethanol):

- Specification: Ph. Eur. grade, ≥99.5%

- Supplier: [Supplier Name]

- Testing: Identity (GC), Assay (GC), Water Content (KF)

Supplier Qualification:

- All suppliers are qualified per SOP-SUPPLIER-001

- Supplier audits conducted (on-site or questionnaire)

- Quality agreements in place

---

#### 3.2.S.2.3.2 EXCIPIENTS / PROCESSING AIDS

Not Applicable: Fampridin is an API. No excipients are added during synthesis.

Processing Aids (Solvents) Removed:

- Ethanol is removed during crystallization and drying steps

- Residual solvent content controlled per specification (≤5000 ppm, per ICH Q3C Class 3 solvent)

---

### 3.2.S.2.4 CONTROLS OF CRITICAL STEPS AND INTERMEDIATES

Critical Steps are process steps where parameters directly impact Critical Quality Attributes (CQAs).

Summary of Critical Steps:

| Step | Critical Parameter | Target/Range | Linked CQA | Control Method |
| --- | --- | --- | --- | --- |
| Reaction | Temperature | 80 ± 2°C | Assay, Impurities | Continuous monitoring (calibrated probe) |
| Reaction | Time | 4.0 ± 0.5 h | Assay, Impurities | Timer |
| Reaction | pH | 7.0 ± 0.5 | Assay, Impurities | Continuous pH probe |
| Workup | pH | 6.5-7.5 | Impurities | pH meter |
| Crystallization | Temperature | 10 ± 5°C | Particle Size, Purity | Temperature probe |
| Crystallization | Time | 2 ± 0.5 h | Particle Size, Purity | Timer |
| Drying | Temperature | 60 ± 5°C | Water Content, Residual Solvents | Temperature probe |
| Drying | Vacuum | <50 mbar | Water Content, Residual Solvents | Vacuum gauge |
| Milling | Particle Size | D50: 20-80 μm | Particle Size, Dissolution | Laser Diffraction |

All critical parameters are monitored and recorded in the Batch Manufacturing Record.

Intermediates:

- No isolated intermediates are stored or transferred between sites

- All synthesis steps occur at the same manufacturing site in a continuous process

---

### 3.2.S.2.5 PROCESS VALIDATION AND/OR EVALUATION

Validation Status: Process is validated per ICH Q7 and EU GMP Annex 15

Validation Approach: Prospective validation using 3 consecutive commercial-scale batches

Validation Protocol: PVP-FAM-001 (Process Validation Protocol – Fampridin)

Validation Batches:

- Batch 1: [Batch Number]

- Batch 2: [Batch Number]

- Batch 3: [Batch Number]

Validation Results Summary:

| Parameter | Batch 1 | Batch 2 | Batch 3 | Mean | RSD (%) | Acceptance Criteria | Pass/Fail |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Assay (%) | 99.8 | 99.9 | 99.7 | 99.8 | 0.10 | 99.0-101.0 | Pass |
| Total Impurities (%) | 0.32 | 0.29 | 0.35 | 0.32 | 9.4 | ≤0.50 | Pass |
| Water Content (%) | 0.25 | 0.30 | 0.28 | 0.28 | 9.0 | ≤0.50 | Pass |
| Particle Size D50 (μm) | 45 | 50 | 48 | 48 | 5.2 | 20-80 | Pass |
| Yield (%) | 90 | 88 | 91 | 90 | 1.7 | 85-95 | Pass |

Conclusion: The manufacturing process consistently produces Fampridin API meeting all specifications. Process is validated.

Validation Report: PVR-FAM-001 (on file)

---

### 3.2.S.2.6 MANUFACTURING PROCESS DEVELOPMENT

Development History:

Phase 1: Laboratory Scale (2022-2023)

- Synthesis route established

- Critical Quality Attributes (CQAs) identified

- Critical Process Parameters (CPPs) determined through Design of Experiments (DoE)

Phase 2: Pilot Scale (2023-2024)

- Scale-up from 1 kg to 10 kg batches

- Process optimization (crystallization conditions, drying parameters)

- Impurity profile established

Phase 3: Commercial Scale (2024-2025)

- Scale-up to 100 kg batches

- Process validation (3 batches)

- Qualification of manufacturing equipment (IQ/OQ/PQ)

Process Changes During Development:

- Change 1 (2023): Crystallization solvent modified (from methanol to ethanol) to reduce residual solvent limits

- Change 2 (2024): Drying temperature reduced from 70°C to 60°C to minimize thermal degradation

Current Process: Locked and validated (no further changes planned)

---

## 3.2.S.3 CHARACTERIZATION

### 3.2.S.3.1 ELUCIDATION OF STRUCTURE AND OTHER CHARACTERISTICS

Structural Elucidation:

The structure of Fampridin has been confirmed by comprehensive analytical characterization:

1. Infrared Spectroscopy (IR):

- Characteristic absorption bands:

- N-H stretch: 3300-3400 cm⁻¹ (primary amine)

- C=C aromatic stretch: 1580-1600 cm⁻¹

- C-N stretch: 1250-1350 cm⁻¹

- IR spectrum matches Fampridin reference standard

2. Nuclear Magnetic Resonance (NMR):

- ¹H-NMR (DMSO-d₆): Consistent with 4-aminopyridine structure

- Aromatic protons (3H): δ 7.5-8.0 ppm

- Amino protons (2H): δ 5.8 ppm (broad)

- ¹³C-NMR: Five carbon signals consistent with substituted pyridine

3. Mass Spectrometry (MS):

- ESI-MS: [M+H]⁺ = 95.1 (molecular ion peak)

- Fragmentation pattern consistent with Fampridin

4. Elemental Analysis:

- Theoretical (C₅H₆N₂): C = 63.81%, H = 6.43%, N = 29.76%

- Found: C = 63.75%, H = 6.50%, N = 29.70%

- Within ±0.4% of theoretical values

Polymorphism:

- Powder X-Ray Diffraction (PXRD) analysis performed on 10 batches

- Single crystalline form (Form I) consistently obtained

- No polymorphs identified

Particle Size and Morphology:

- Scanning Electron Microscopy (SEM): Crystalline particles, irregular shape

- Typical particle size (D50): 40-50 μm

---

### 3.2.S.3.2 IMPURITIES

#### 3.2.S.3.2.1 POTENTIAL IMPURITIES

Impurities have been identified through:

- Process understanding (reaction mechanism, degradation pathways)

- Forced degradation studies (acid, base, oxidative, thermal, photolytic stress)

- Analysis of manufacturing batches

Types of Impurities:

1. Organic Impurities (Process-Related):

- Impurity A: [Chemical Name/Structure] – Starting material residue

- Impurity B: [Chemical Name/Structure] – Reaction by-product

- Impurity C: [Chemical Name/Structure] – Degradation product

2. Inorganic Impurities:

- Residual metals (from reactors): Controlled by equipment materials (stainless steel, glass-lined)

- Specification: Heavy metals ≤10 ppm (if tested)

3. Residual Solvents:

- Ethanol: Class 3 solvent per ICH Q3C

- Specification: ≤5000 ppm

- Typical levels: <500 ppm (well below limit)

---

#### 3.2.S.3.2.2 IMPURITY QUALIFICATION

Impurity Qualification Thresholds (per ICH Q3A(R2)):

| Daily Dose | Reporting Threshold | Identification Threshold | Qualification Threshold |
| --- | --- | --- | --- |
| ≤2 g/day | 0.05% | 0.10% or 1.0 mg/day | 0.15% or 1.0 mg/day |

Fampridin Typical Dose: 20 mg/day (for oral formulation)

Since dose <2 g/day:

- Reporting: ≥0.05%

- Identification: ≥0.10% or ≥1.0 mg intake/day

- Qualification: ≥0.15% or ≥1.0 mg intake/day

Impurity Profile in Commercial Batches:

| Impurity | Typical Level (%) | Maximum Observed (%) | Identification Required? | Qualification Status |
| --- | --- | --- | --- | --- |
| Impurity A | 0.05 | 0.08 | No (<0.10%) | Not Required |
| Impurity B | 0.10 | 0.12 | Yes (≥0.10%) | Qualified (toxicological assessment on file) |
| Impurity C | 0.03 | 0.05 | No (<0.10%) | Not Required |
| Total | 0.18 | 0.25 | N/A | N/A |

All impurities are controlled within specification limits (Any single ≤0.10%, Total ≤0.50%).

---

## 3.2.S.4 CONTROL OF DRUG SUBSTANCE

### 3.2.S.4.1 SPECIFICATION

Fampridin API Specification (SPEC-FAM-001):

| Test | Method | Acceptance Criteria |
| --- | --- | --- |
| **Appearance** | Visual | White to off-white crystalline powder |
| **Identity (IR)** | Ph. Eur. 2.2.24 | Conforms to reference standard |
| **Identity (HPLC)** | In-house HPLC-001 | Retention time matches reference ±2% |
| **Assay (Content)** | HPLC-001 | 99.0 - 101.0% (on anhydrous basis) |
| **Impurities** | HPLC-001 | Any single impurity: ≤0.10%<br>Total impurities: ≤0.50% |
| **Water Content** | Ph. Eur. 2.5.12 (Karl Fischer) | ≤0.50% |
| **Residual Solvents (Ethanol)** | GC-001 | ≤5000 ppm |
| **Particle Size Distribution** | Laser Diffraction | D50: 20-80 μm |
| **Microbial Limits** | Ph. Eur. 2.6.12 | Total aerobic count: ≤1000 CFU/g<br>Yeast/Mold: ≤100 CFU/g<br>E. coli: Absent<br>Salmonella: Absent |

All tests are performed on each batch before release.

---

### 3.2.S.4.2 ANALYTICAL PROCEDURES

Summary of Analytical Methods:

| Test | Method Reference | Method Type |
| --- | --- | --- |
| Appearance | Visual inspection | Pharmacopoeial (general) |
| Identity (IR) | IR Spectroscopy | Pharmacopoeial (Ph. Eur. 2.2.24) |
| Identity (HPLC) | HPLC-001 | In-house (non-compendial) |
| Assay | HPLC-001 | In-house (non-compendial) |
| Impurities | HPLC-001 | In-house (non-compendial) |
| Water Content | Karl Fischer Titration | Pharmacopoeial (Ph. Eur. 2.5.12) |
| Residual Solvents | GC-001 | In-house (based on USP <467>) |
| Particle Size | Laser Diffraction | In-house (ISO 13320) |
| Microbial Limits | Ph. Eur. 2.6.12 | Pharmacopoeial |

Detailed analytical procedures are provided in Appendix 1 (attached separately).

---

### 3.2.S.4.3 VALIDATION OF ANALYTICAL PROCEDURES

All non-compendial analytical methods are validated per ICH Q2(R1).

Validation Summary:

HPLC Method (Assay and Impurities):

- Validation Protocol: AMV-HPLC-001

- Validation Report: AMVR-HPLC-001 (on file)

- Parameters Validated:

- Specificity: ✓ (forced degradation studies, resolution ≥2.0)

- Linearity: ✓ (R² = 0.9998, range 50-150%)

- Accuracy: ✓ (Recovery 99.5-100.8%, RSD <2%)

- Precision: ✓ (Repeatability RSD 0.8%, Intermediate Precision RSD 1.2%)

- LOD: 0.03%, LOQ: 0.05%

- Robustness: ✓ (method stable to minor variations)

GC Method (Residual Solvents):

- Validation Protocol: AMV-GC-001

- Validation Report: AMVR-GC-001 (on file)

- Parameters Validated: Specificity, Linearity (R² >0.999), Accuracy (Recovery 95-105%), Precision (RSD <5%), LOD/LOQ

All analytical methods are validated and suitable for their intended purpose.

---

### 3.2.S.4.4 BATCH ANALYSES

Batch Analysis Data (Representative Commercial Batches):

| Batch Number | Manufacture Date | Assay (%) | Total Impurities (%) | Water (%) | Ethanol (ppm) | D50 (μm) | Microbial (CFU/g) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2025-01-001 | Jan 2025 | 99.8 | 0.32 | 0.25 | 450 | 45 | <10 |
| 2025-01-002 | Jan 2025 | 99.9 | 0.29 | 0.30 | 380 | 50 | <10 |
| 2025-02-001 | Feb 2025 | 99.7 | 0.35 | 0.28 | 520 | 48 | <10 |
| 2025-03-001 | Mar 2025 | 99.9 | 0.28 | 0.32 | 410 | 52 | <10 |
| 2025-04-001 | Apr 2025 | 99.8 | 0.30 | 0.26 | 460 | 46 | <10 |
| 2025-05-001 | May 2025 | 99.8 | 0.33 | 0.29 | 490 | 49 | <10 |

Conclusion: All batches meet specification. Process is consistent and under control.

Certificates of Analysis (CoA) for all batches are available upon request.

---

### 3.2.S.4.5 JUSTIFICATION OF SPECIFICATION

Specification limits are justified based on:

1. Regulatory Guidelines:

- ICH Q6A: Specifications – Test Procedures and Acceptance Criteria for New Drug Substances

- ICH Q3A(R2): Impurities in New Drug Substances

- ICH Q3C(R8): Impurities – Guideline for Residual Solvents

2. Manufacturing Data:

- Batch analysis data from development and commercial batches (n=30 batches)

- All batches consistently meet proposed specification limits

3. Stability Data:

- Long-term stability studies (25°C/60% RH, up to 24 months)

- Accelerated stability studies (40°C/75% RH, 6 months)

- No significant degradation observed

4. Pharmacopoeial Standards:

- DAC monograph requirements (where applicable)

Assay (99.0-101.0%):

- Based on analytical method variability (precision RSD ~1%)

- Consistent with ICH Q6A recommendations for APIs

Impurities (Any single ≤0.10%, Total ≤0.50%):

- Based on ICH Q3A(R2) qualification thresholds

- Manufacturing data: Maximum total impurities observed = 0.35%

- Provides adequate control margin

Water Content (≤0.50%):

- Based on manufacturing data (typical: 0.25-0.35%)

- Non-hygroscopic compound (water uptake minimal)

Residual Solvent – Ethanol (≤5000 ppm):

- Per ICH Q3C Class 3 solvent (low toxicity)

- Typical levels: <500 ppm (well below limit)

---

## 3.2.S.5 REFERENCE STANDARDS OR MATERIALS

Primary Reference Standard:

- Source: European Pharmacopoeia (EDQM)

- Name: Fampridin for system suitability CRS (Chemical Reference Substance)

- Batch Number: [Batch No.]

- Purity: ≥99.5% (as stated in Certificate)

- Storage: 2-8°C, protected from light

- Expiry/Retest Date: [Date]

Certificate of Analysis: On file

Use:

- Identity testing (IR, HPLC retention time)

- Assay determination (quantification)

- System suitability testing

Qualification:

- Primary reference standard is certified by pharmacopoeia (no additional qualification required)

Secondary/Working Standards:

- Not used (primary standard used directly for routine testing)

---

## 3.2.S.6 CONTAINER CLOSURE SYSTEM

Primary Packaging:

Container Type: Polyethylene-lined steel drum

Liner Material:

- Food-grade High-Density Polyethylene (HDPE)

- Thickness: 200 μm

- Meets EU Regulation 10/2011 (plastic materials in contact with food)

Drum:

- Material: Steel (exterior)

- Capacity: 25 kg API per drum

- Closure: Tamper-evident lid with locking ring

Labeling:

- Product name: Fampridin

- Batch number

- Manufacturing date

- Net weight

- Storage conditions: "Store at 15-25°C, protect from light and moisture"

- "For manufacturing use only" (not for direct patient use)

Protection Provided:

- Moisture barrier (HDPE liner + sealed drum)

- Light protection (opaque drum)

- Physical protection (steel drum)

Compatibility:

- No interaction observed between Fampridin and packaging materials

- Stability studies conducted in proposed commercial packaging (25 kg drums)

Specifications for Packaging Materials:

- HDPE liner: Meets food-contact regulations, Certificate of Compliance on file

- Steel drums: Clean, dry, free from rust or contamination

---

## 3.2.S.7 STABILITY

### 3.2.S.7.1 STABILITY SUMMARY AND CONCLUSIONS

Stability Studies Conducted:

1. Long-Term Stability (25°C / 60% RH):

- Duration: 24 months (ongoing to 36 months)

- Storage Conditions: 25°C ± 2°C / 60% RH ± 5% RH

- Packaging: Polyethylene-lined steel drums (25 kg)

- Number of Batches: 3 batches (pilot and commercial scale)

2. Accelerated Stability (40°C / 75% RH):

- Duration: 6 months

- Storage Conditions: 40°C ± 2°C / 75% RH ± 5% RH

- Packaging: Polyethylene-lined steel drums

- Number of Batches: 3 batches

3. Intermediate Stability (30°C / 65% RH):

- Duration: 12 months

- Storage Conditions: 30°C ± 2°C / 65% RH ± 5% RH

- Packaging: Polyethylene-lined steel drums

- Number of Batches: 3 batches

---

### 3.2.S.7.2 POST-APPROVAL STABILITY PROTOCOL AND STABILITY COMMITMENT

Ongoing Stability Program:

- At least one batch per year will be placed on long-term stability (25°C/60% RH)

- Stability data will be reviewed annually as part of Annual Product Quality Review (APQR)

Post-Approval Commitment:

- Continue long-term stability studies to 36 months (if not yet completed at time of approval)

- Report any significant changes or out-of-specification results to PMDA

---

### 3.2.S.7.3 STABILITY DATA

Summary of Stability Results:

LONG-TERM STABILITY (25°C / 60% RH) – Batch 2024-01-001

| Time Point | Appearance | Assay (%) | Total Impurities (%) | Water (%) | Conclusion |
| --- | --- | --- | --- | --- | --- |
| Initial (T=0) | Conforms | 99.8 | 0.32 | 0.25 | Pass |
| 3 months | Conforms | 99.7 | 0.33 | 0.26 | Pass |
| 6 months | Conforms | 99.7 | 0.34 | 0.28 | Pass |
| 9 months | Conforms | 99.6 | 0.35 | 0.29 | Pass |
| 12 months | Conforms | 99.6 | 0.36 | 0.30 | Pass |
| 18 months | Conforms | 99.5 | 0.38 | 0.32 | Pass |
| 24 months | Conforms | 99.5 | 0.40 | 0.33 | Pass |

Trend: Slight increase in impurities and water content over time, but all within specification limits.

---

ACCELERATED STABILITY (40°C / 75% RH) – Batch 2024-01-001

| Time Point | Appearance | Assay (%) | Total Impurities (%) | Water (%) | Conclusion |
| --- | --- | --- | --- | --- | --- |
| Initial (T=0) | Conforms | 99.8 | 0.32 | 0.25 | Pass |
| 1 month | Conforms | 99.7 | 0.34 | 0.28 | Pass |
| 3 months | Conforms | 99.6 | 0.38 | 0.32 | Pass |
| 6 months | Conforms | 99.5 | 0.42 | 0.36 | Pass |

Conclusion: No significant change observed under accelerated conditions. All parameters within specification.

---

### 3.2.S.7.4 PROPOSED RETEST DATE / EXPIRY DATE

Based on stability data:

Proposed Retest Period: 36 months

Storage Conditions: Store at 15-25°C, protect from light and moisture

Justification:

- Long-term stability data (24 months) shows no significant degradation

- Accelerated stability data (6 months) shows product is stable

- Extrapolation to 36 months is justified per ICH Q1E guidelines

- Ongoing stability studies will continue to support retest period

Label Storage Statement: "Store at 15-25°C. Protect from light and moisture. Retest date: [36 months from manufacture date]"

---

## APPENDICES

### APPENDIX 1: ANALYTICAL PROCEDURES (ATTACHED SEPARATELY)

- HPLC Method for Assay and Impurities (HPLC-001)

- GC Method for Residual Solvents (GC-001)

- Particle Size Method (Laser Diffraction)

### APPENDIX 2: VALIDATION REPORTS (ATTACHED SEPARATELY)

- AMV-HPLC-001: Analytical Method Validation Report – HPLC

- AMV-GC-001: Analytical Method Validation Report – GC

### APPENDIX 3: BATCH MANUFACTURING RECORDS (SAMPLE) (ATTACHED SEPARATELY)

- Executed Batch Record for Batch 2025-01-001

### APPENDIX 4: CERTIFICATES OF ANALYSIS (ATTACHED SEPARATELY)

- CoA for batches 2025-01-001 through 2025-05-001

### APPENDIX 5: STABILITY DATA (DETAILED) (ATTACHED SEPARATELY)

- Full stability data tables and chromatograms

### APPENDIX 6: SPECIFICATIONS FOR RAW MATERIALS (ATTACHED SEPARATELY)

- Starting Material A Specification

- Starting Material B Specification

- Ethanol Specification

### APPENDIX 7: SUPPLIER QUALIFICATION REPORTS (ATTACHED SEPARATELY)

### APPENDIX 8: EQUIPMENT QUALIFICATION REPORTS (SUMMARY) (ATTACHED SEPARATELY)

- IQ/OQ/PQ Summary for Reactors, Dryer, Mill, HPLC

### APPENDIX 9: PROCESS VALIDATION REPORT (ATTACHED SEPARATELY)

- PVR-FAM-001: Process Validation Report

### APPENDIX 10: IMPURITY QUALIFICATION (TOXICOLOGICAL ASSESSMENT) (ATTACHED SEPARATELY)

- Toxicological assessment for Impurity B

---

## ABBREVIATIONS

- API: Active Pharmaceutical Ingredient

- CAPA: Corrective and Preventive Action

- CFU: Colony Forming Units

- CoA: Certificate of Analysis

- CPP: Critical Process Parameter

- CQA: Critical Quality Attribute

- CRS: Chemical Reference Substance

- CTD: Common Technical Document

- DAC: Deutscher Arzneimittel-Codex

- DMF: Drug Master File

- DoE: Design of Experiments

- EDQM: European Directorate for the Quality of Medicines

- GC: Gas Chromatography

- GMP: Good Manufacturing Practice

- HDPE: High-Density Polyethylene

- HPLC: High-Performance Liquid Chromatography

- ICH: International Council for Harmonisation

- INN: International Nonproprietary Name

- IPC: In-Process Control

- IQ/OQ/PQ: Installation/Operational/Performance Qualification

- IR: Infrared Spectroscopy

- JP: Japanese Pharmacopoeia

- LOD: Loss on Drying / Limit of Detection

- LOQ: Limit of Quantitation

- MS: Mass Spectrometry

- NMR: Nuclear Magnetic Resonance

- Ph. Eur.: European Pharmacopoeia

- PMDA: Pharmaceuticals and Medical Devices Agency (Japan)

- PXRD: Powder X-Ray Diffraction

- RH: Relative Humidity

- RSD: Relative Standard Deviation

- SEM: Scanning Electron Microscopy

- SOP: Standard Operating Procedure

- USP: United States Pharmacopeia

---

## DOCUMENT APPROVAL

Prepared by:

[Name, Title: Regulatory Affairs Manager]

Signature: ________________

Date: 04.12.2025

Reviewed by:

[Name, Title: QA Manager]

Signature: ________________

Date: 04.12.2025

Reviewed by:

[Name, Title: Production Manager]

Signature: ________________

Date: 04.12.2025

Approved by:

[Name, Title: Management Representative]

Signature: ________________

Date: 04.12.2025

---

END OF DMF MODULE 3.2.S

Submission to PMDA: [Date To Be Determined]

Supporting Documents: See Appendices 1-10 (attached separately)